# Software Documentation

### 1. Software Overview

- What does the software do?

The software is used to control the movement of the doll, it uses functions to define the possible paths that the

- Platforms used:

MicroPython, MIT App Inventor

### 2. System Logic

Explain flow in simple steps:

- User presses OK button

- App sends the sequence via Bluetooth

- ESP32 receives data (the sequence of the points for the moving of the doll)

- User presses START button to begin the sequence

- The ESP32 receives the sequence

- It separates each character int the sequence so that the code can identify each number as a different point’s

- It converts the sequence to a series of transitions from the transition table (which defines the movements of the 2 dc motors using functions to move the doll’s wheels).

- The IR Sensor continuously senses the obstacle below the doll

- If it detects an obstacle, then the doll moves forward.

- The moment the doll stops detecting a surface underneath it, it moves backward and stops.

### 3. Flowchart

### 4. App Flow (If MIT App Inventor)

4. App Flow (If MIT App Inventor)

User clicks on “Scan for devices” to start scanning for bluetooth device

User clicks on “Dancing Doll”  bluetooth and presses “connect” button

Status shows: Connected to Dancing Doll

User presses Go! button

Interfaces switches to stage layout - by setting visible commands of Bluetooth connecting components to false, and visible command of the canvas stage layout to true

User clicks one point, then another, doing so for all 4 points connecting them - when each point is clicked a line is drawn connecting the points visually as well.

-> layout for points was created by trial and error mapping of x, y coordinates to find a threshold for each point - so when the point and a small area near it is clicked, it registers the touch as a specific number in the sequence

As the points are clicked, the Sequence is formed and shown on the MIT interface.

-> global sequence variable was used in order to create a sequence of x,y coordinates and store the value + feed it into the esp32 micro python code

User presses “start” and the sequence gets sent to the esp32 to and then executes it based on the micropython code

### 5. Software  Code Iterations

ITERATION 1

def move_3():

motor1.on()

motor2.off()

time.sleep(1)

stop()

def move_4():

motor2.on()

motor1.off()

time.sleep(1)

stop()

def move_5():

motor1.on()

motor2.on()

time.sleep(1)

stop()

def move_6():

motor1.off()

motor2.on()

time.sleep(1)

stop()

def choreography(seq_list):

for m in seq_list:

if m == "1":

move_1()

elif m == "2":

move_2()

elif m == "3":

move_3()

elif m == "4":

move_4()

elif m == "5":

move_5()

elif m == "6":

move_6()

time.sleep(0.2)

from machine import Pin

import bluetooth

import time

motor1 = Pin(5, Pin.OUT)

motor2= Pin(19, Pin.OUT)

value = ""

name = "Dancer"

ble = bluetooth.BLE()

ble.active(False)

time.sleep(0.5)

ble.active(True)

ble.config(gap_name=name)

service_UUID = bluetooth.UUID("6e400001-b5a3-f393-e0a9-e50e24dcca9e")

char_UUID = bluetooth.UUID("6e400002-b5a3-f393-e0a9-e50e24dcca9e")

char = (char_UUID, bluetooth.FLAG_WRITE)

service = (service_UUID, (char,),)

((char_handle,),) = ble.gatts_register_services((service,))

def stop():

motor1.off()

motor2.off()

def move_1():

motor1.on()

time.sleep(1)

stop()

def move_2():

motor2.on()

time.sleep(1)

stop()

name_bytes = device_name.encode()

flags = bytearray([0x02, 0x01, 0x06])

short_name = bytearray([len(name_bytes) + 1, 0x08]) + name_bytes

full_name = bytearray([len(name_bytes) + 1, 0x09]) + name_bytes

adv_data = flags + short_name + full_name

ble.gap_advertise(50, adv_data=adv_data)

print("Advertising as:", device_name)

advertise(name)

ble.irq(event_occured)

while True:

if value:

print("Dancing!")

choreography(value)

value = ""

time.sleep(0.2)

def event_occured(event, data):

if event == 1:

print("Connected")

elif event == 2:

print("Disconnected")

advertise(name)

elif event == 3:

conn_handle, value_handle = data

if value_handle == char_handle:

raw_msg = ble.gatts_read(char_handle).rstrip(b'\x00')

msg = raw_msg.decode().strip()

print("Received:", msg)

global value

value = msg

def advertise(device_name):

FINAL CODE

from machine import Pin, PWM

import bluetooth

import time

my_ir_sensor = Pin(15, Pin.IN, Pin.PULL_UP)

LEFT_IN1 = Pin(26, Pin.OUT)

LEFT_IN2 = Pin(27, Pin.OUT)

LEFT_ENA = PWM(Pin(25))

LEFT_ENA.freq(1000)

RIGHT_IN1 = Pin(14, Pin.OUT)

RIGHT_IN2 = Pin(12, Pin.OUT)

RIGHT_ENB = PWM(Pin(13))

RIGHT_ENB.freq(1000)

SPEED = 500

IN1 = Pin(18, Pin.OUT)

IN2 = Pin(19, Pin.OUT)

IN3 = Pin(21, Pin.OUT)

IN4 = Pin(22, Pin.OUT)

value = ""

sequence = []

current_pos = 'home'

name = "DancerBot"

def stop():

LEFT_IN1.off(); LEFT_IN2.off()

RIGHT_IN1.off(); RIGHT_IN2.off()

LEFT_ENA.duty(0)

RIGHT_ENB.duty(0)

def forward():

LEFT_IN1.on(); LEFT_IN2.off()

RIGHT_IN1.on(); RIGHT_IN2.off()

LEFT_ENA.duty(SPEED); RIGHT_ENB.duty(SPEED)

def backward():

LEFT_IN1.off(); LEFT_IN2.on()

RIGHT_IN1.off(); RIGHT_IN2.on()

LEFT_ENA.duty(SPEED); RIGHT_ENB.duty(SPEED)

def turn_left():

LEFT_IN1.off(); LEFT_IN2.on()

RIGHT_IN1.on(); RIGHT_IN2.off()

LEFT_ENA.duty(SPEED); RIGHT_ENB.duty(SPEED)

def turn_right():

LEFT_IN1.on(); LEFT_IN2.off()

RIGHT_IN1.off(); RIGHT_IN2.on()

LEFT_ENA.duty(SPEED); RIGHT_ENB.duty(SPEED)

def stepper_step():

IN1.on(); IN2.off(); IN3.off(); IN4.off()

time.sleep(0.002)

IN1.off(); IN2.on(); IN3.off(); IN4.off()

time.sleep(0.002)

IN1.off(); IN2.off(); IN3.on(); IN4.off()

time.sleep(0.002)

IN1.off(); IN2.off(); IN3.off(); IN4.on()

time.sleep(0.002)

def spin():

for i in range(80):

stepper_step()

def do_move(steps):

for step in steps:

direction = step[0]

duration = step[1]

if direction == "forward":

forward()

elif direction == "backward":

backward()

elif direction == "left":

turn_left()

elif direction == "right":

turn_right()

time.sleep(duration)

stop()

spin()

time.sleep(0.2)

TRANSITIONS = {

('home', 1): [('forward', 2)],

('home', 2): [('forward', 1), ('right', 0.6), ('forward', 1.5)],

('home', 3): [('left', 0.7), ('forward', 2)],

('home', 4): [('right', 0.7), ('forward', 2)],

(1, 2): [('right', 0.6), ('forward', 1.5)],

(1, 3): [('left', 0.9), ('forward', 2)],

(1, 4): [('right', 0.9), ('forward', 2)],

(2, 1): [('left', 0.6), ('forward', 1.5)],

(2, 3): [('left', 1.2), ('forward', 2)],

(2, 4): [('forward', 1), ('right', 0.5), ('forward', 0.8)],

(3, 1): [('right', 0.9), ('forward', 2)],

(3, 2): [('right', 1.2), ('forward', 2)],

(3, 4): [('right', 1.8), ('forward', 1.5)],

(4, 1): [('left', 0.9), ('forward', 2)],

(4, 2): [('forward', 0.8), ('left', 0.5), ('forward', 1)],

(4, 3): [('left', 1.8), ('forward', 1.5)],

(1, 'home'): [('backward', 2)],

(2, 'home'): [('backward', 1.5), ('left', 0.6), ('backward', 1)],

(3, 'home'): [('backward', 2), ('right', 0.7)],

(4, 'home'): [('backward', 2), ('left', 0.7)],

}

def return_home():

global current_pos

if current_pos == 'home':

return

key = (current_pos, 'home')

if key in TRANSITIONS:

print("Returning home")

do_move(TRANSITIONS[key])

current_pos = 'home'

print("Doll is back at home.")

def run_sequence(seq):

global current_pos

if not seq:

print("Empty sequence received.")

return

print("Starting sequence:", seq)

for next_point in seq:

if next_point == current_pos:

print("Skipping duplicate point:", next_point)

continue

key = (current_pos, next_point)

if key in TRANSITIONS:

print("Moving:", current_pos, "->", next_point)

do_move(TRANSITIONS[key])

current_pos = next_point

time.sleep(0.5)

spin()

else:

print("ERROR: No transition for", key)

break

spin()

return_home()

stop()

ble = bluetooth.BLE()

ble.active(True)

ble.config(gap_name=name)

service_UUID = bluetooth.UUID("6e400001-b5a3-f393-e0a9-e50e24dcca9e")

char_UUID = bluetooth.UUID("6e400002-b5a3-f393-e0a9-e50e24dcca9e")

char = (char_UUID, bluetooth.FLAG_WRITE)

service = (service_UUID, (char,))

((char_handle,),) = ble.gatts_register_services((service,))

def event_occurred(event, data):

global value

if event == 3:

conn_handle, value_handle = data

if value_handle == char_handle:

raw_msg = ble.gatts_read(char_handle).rstrip(b'\x00')

msg = raw_msg.decode().strip()

print("Received:", msg)

value = msg

def advertise(device_name):

name_bytes = device_name.encode()

flags = bytearray([0x02, 0x01, 0x06])

full_name = bytearray([len(name_bytes) + 1, 0x09]) + name_bytes

adv_data = flags + full_name

ble.gap_advertise(100000, adv_data=adv_data)

print("Advertising as:", device_name)

advertise(name)

ble.irq(event_occurred)

print("Doll is ready. Build sequence then press START.")

while True:

ir_values = my_ir_sensor.value()

if value:

msg = value

value = ""

if msg == "CLEAR":

if ir_value=1:

sequence.clear()

print("Sequence cleared.")

else:

if ir_value=1:

parts = msg.split(",")

sequence = []

for p in parts:

if p != "":

try:

sequence.append(int(p.strip()))

except:

print("Invalid number in sequence:", p)

print("Sequence saved:", sequence)

if sequence:

run_sequence(sequence)

sequence.clear()

else:

print("No sequence set")

else:

stop()

time.sleep(0.2)